ASSIGNMENT # 10

Rankine

Cycle

EXPERIMENT # 9:

Investigate the performance of Rankine cycle

Introduction:

- The Rankine cycle or Rankine Vapor Cycle is the process widely used by power plants such as coal-fired power plants or nuclear reactors. In this mechanism, a fuel is used to produce heat within a boiler, converting water into steam which then expands through a turbine producing useful work.

- The Rankine cycle is a model used to predict the performance of steam turbine systems. It was also used to study the performance of reciprocating steam engines. The Rankine cycle is an idealized thermodynamic cycle of a heat engine that converts heat into mechanical work while undergoing phase change as shown in figure # 8-1.

Diagram:  figure  #  8-1:

Basic Principle:

- The Rankine cycle is an idealized thermodynamic cycle of a heat engine that converts heat into mechanical work while undergoing phase change. ... The heat is supplied externally to a closed loop, which usually uses water as the working fluid.

Parts:

- Steam Pressure

- Condenser Pressure

- Tachometer

- Temperature Indicator

- Break Load Indicator

- Break Cooling water control

- Steam Solenoid Valve

- Steam Outlet After Boiler

- Throttle Valve

- Belt Break

- Tachometer Optical Sensor

- Impulse Turbine

- Break Cooling water outlet

- Three pipe steam

- Gauge

- Boiler Pressure

- Pressure Leave valve

- Boiler leave indicator

- Nozzle

Parts Explanation:

Tachometer:                                                                            Figure # 8-2:

- Tachometer is a device which is used to measure the speed of the turbine. It allows turbine rotational speed to be evaluated.

- A tachometer (revolution-counter, tach, rev-counter, RPM gauge) is an instrument measuring the rotation speed of a shaft or disk, as in a motor or other machine. The device usually displays the revolutions per minute (RPM) on a calibrated analogue dial, but digital displays are increasingly common as shown in figure  # 8-2:

Temperature Indicator:

- Nvis 607 is a simple digital general purpose

temperature indicator designed indicate temperature using a sensor of K-type.

- Due to the fact that there are different physical measuring principles depending on the sensor, diverse temperature indicators will be required. Pt100 resistance sensors can get changed with temperature and resistance. Temperature indicators should determine the current resistance as shown in figure # 8-3:

Figure # 8-3:

Break Load Indicator:

- Steam flow to the turbine can be throttled by a hand                    Figure # 8-4:

valve and the boiler turbine inlet and condenser pressures are indicated on gauges. The impulse turbines driven by a convergent divergent nozzle and turns a brake wheel with speed sensor and digital indicator allowing true shaft power to be determined as shown in figure # 8-4.

Break Cooling water Control:

Those massive draw works disc brakes get hot. ... Their main          Figure # 8-5:

purpose is to control the speed of rotation by turning

rotational energy into heat energy

. ... Additionally, if yours has a water cooling system.

- The brakes are pneumatically actuated and cooled down through the use of a continuous flow of coolant from either an open or closed loop cooling system. Water-cooled brakes are found in a variety of industries and applications as shown in figure # 8-5.

Steam Solenoid Valve:                                                             Figure # 8-6:

- Steam Solenoid Valves. The name given as

solenoid valve to the electromagnetic valve used to control fluids such as water,air, steam, gas etc. ... When the electric current moves in the coil, the piston becomes the electromagnet and moves the piston according to the state of use of the coil as shown in figure # 8-6.

Throttle Valve:                                                                                Figure # 8-7:

- Designed to control steam flow and prevent catastrophic failure, a trip and throttle (T&T) valve is a critical safety device on a steam turbine. The reliability of a T&T valve is essential to ensure the safe operation of the steam turbine and the safety of personnel working around the equipment as shown

in figure # 8-7:

Impulse Turbine:                                                                   Figure # 8-8:

- An impulse turbine is a turbine that is driven by high velocity jets of water or steam from a nozzle directed on to vanes or buckets attached to a wheel. The resulting impulse (as described by Newton's second law of motion) spins the turbine and removes kinetic energy from the fluid flow

as shown in figure # 8-8:

Gauge:

- A gauge or gage, in science and engineering,

is a device used to make measurements or in                                       Figure # 8-9:

order to display certain dimensional information.

- A wide variety of tools exist which serve such functions, ranging from simple pieces of material against which sizes can be measured to complex pieces of machinery

as shown in figure # 8-9:

Safety Valve:

- A safety valve is a valve that acts as a fail-safe. An example of safety valve is a pressure relief ... power generation, food, drinks, cosmetics and pharmaceuticals industries, the term safety valve is associated with the terms pressure relief valve .

Boiler Level Indicator:                                                          Figure # 8-10:

The level of water in a steam boiler must be                                                                                                  carefully controlled, to ensure good quality steam is ... In most cases, the simple gauge glass on the steam / water drum or boiler shell is used as the indicator. ... The boiler steam generation rating.

- As we know that simple Rankin Cycle is used in Power Plants and boiler is the indispensable part of a power plant. Boiler is the place where water gets converted into steam. Actually water takes up Latent heat of Vaporization from the heat supplied and changes to steam as shown in figure # 8-10:

Nozzle:                                                                                       Figure # 8-11:

- A nozzle is often a pipe or tube of varying cross sectional area and it can be used to direct or modify the flow of a fluid (liquid or gas). Nozzles are frequently used to control the rate of flow, speed, direction, mass, shape, and/or the pressure of the stream that emerges from them.

- A nozzle is a device designed to control the direction or characteristics of a fluid flow as it exits an enclosed chamber or pipe. A nozzle is often a pipe or tube of varying cross sectional area and it can be used to direct or modify the flow of a fluid as shown in figure # 8-11:

Convergent-divergent nozzle:                                   Figure # 8.1-11:

- Suppose a nozzle is used to obtain a supersonic stream starting from low speeds at the inlet. It is clear that the nozzle must converge in the subsonic portion and diverge in the supersonic portion. Such a nozzle is called a convergent-divergent nozzle as shown in figure # 8.1-11:

Explanation:

Figure # 8-12:

Pump:

- Compression of the fluid to high pressure using a pump (this takes work)

(Figure 8-12: Steps 1 to 2)

Boiler:

- The compressed fluid is heated to the final temperature (which is at boiling point), therefore, a phase change occurs—from liquid to vapor. (Figure 8-12: Steps 2 to 3)

Turbine:

- Expansion of the vapor in the turbine. (Figure 8-12: Steps 3 to 4)

Condenser:

- Condensation of the vapor in the condenser (where the waste heat goes to the final heat sink (the atmosphere or a large body of water (lake or river)

(Figure 8-12: Steps 4 to 1)

Process:

Process 1–2:

- The working fluid is pumped from low to high pressure. As the fluid is a liquid at this stage, the pump requires little input energy.

- In other words Process 1-2 is [Isentropic compression in pump]

Process 2–3:

- The high-pressure liquid enters a boiler, where it is heated at constant pressure by an external heat source to become a dry saturated vapor. The input energy required can be easily calculated graphically, using an enthalpy–entropy chart (h–s chart, or Moliere diagram), or numerically, using steam tables.

- Input is that material which is most common in the region or country.

- In other words Process 2-3 is [Constant pressure heat addition in boiler]

Process 3–4:

- The dry saturated vapor expands through a turbine, generating power. This decreases the temperature and pressure of the vapor, and some condensation may occur. The output in this process can be easily calculated using the chart or tables noted above.

- In other words Process 3-4 is [Isentropic expansion in turbine]

Process 4–1:

- The wet vapor then enters a condenser, where it is condensed at a constant pressure to become a saturated liquid by cool water.

Important Points:

- There should be water in boiler.

- There should be cold water at inlet of condenser.

- Before 85 psi, manual valve should be closed.

- At manual valve, pressure should be 85 psi.

- Maximum upper limit pressure of steam is 90 psi.

- Normal pressure of steam is 85 psi.

- Relief valve set pressure is 100 psi.

- Throttle valve should not be completely off. It is because, steam coming quantity is controlled by Throttled valve.

- There are two pipes for steam at top.

- There is only one pipe for water at top.

Important  Points  about  Condenser:

Heat Transfer Area:

- When steam becomes liquid on pipe when touch it. Cold water is present at pipe of Condenser. Because cold water is used to cool the steam and heat is removed from steam and it becomes liquid. This happens when condenser pipe is straighten.

- Heat transfer area is 0.132 m2 .

- Specific heat capacity = Cp = 4.18kJ / kg K

- Surface area = 2πRl

- Q = mCPΔT (change in temperature, heated rod get cool in air)

- Q = hAsΔT ( shape effect, As = heat transfer area )

- Cp  of water > Cp  of air

Important  Points  about  Turbine Rotor:

- Blade pitch circle diameter = 45mm

- No of blades = 45

- Blades inlet = 40o

- Blades outlet = 40o

Break Load:

- Pulley diameter = 40mm

Important  Points  about  Turbine :

- Throat diameter = 1.38mm

- Exit diameter = 3mm

- Discharge angle = 20o

Important  Points  about  Turbine :

- Nominal heat input= 6kW at 220 V (depend upon main voltage )

- Maximum pressure control= 90 Psi

- Normal pressure control= 85 Psi

- Relief Valve= 100 Psi

Determination of torque, power and specific steam consumption:

Torque = Force x Radius

At  force = 0.77N

= 0.77 x 0.023

Torque= 0.01771Nm

At force = 1.03N

Torque = 1.03 x 0.023 = 0.02369Nm

Power:

P = T x W

= T x 2πN/60

At N= 375000RPM and T = 0.01771Nm

= 0.01771 x 2pi x 375000/60

Power= 695.12W

At N = 223000RPM and T = 0.02369Nm

= 0.02369 x 2π 223000/60

Power = 552.94W

Mass Flow Rate:

Mass flow rate = m / t

For mass

V= 200ml= 200 x 10^-3 x 1/1000

= 0.0002m^3

e = m/V

m = e x V

= 1000 x 0.0002

= 0.2

Mass flow rate = 0.2/117

= 0.00017kg/s

Temperature Explanation:

T1 = Turbine inlet temperature

T2 = Condenser outlet temperature

T3 = Cold water at condenser inlet (hot water)

T4 = Hot water at condenser outlet

T5 = Condensed water temperature that is in the Hollow condenser

T6 = Pump inlet (Tube makeup tank)

T7 = Boiler outlet (steam temperature)

T8 = Temperature after manual valve  at outlet of boiler

Efficiency of steam power plant:

- The power plant that operates on coal constitutes almost 41% of the world’s electricity generation.

- It is the modified Rankine thermodynamic cycle on which the coal fired power plant operates.

- The overall efficiency of the coal power plant ranges from 32% to 42%. And this is calculated by the superheat pressures, and super heat and reheat temperatures of steam.

- Most of the large power plants that operates at steam pressures of 170 bar,570 ℃ superheat and  570 ℃ reheat steam temperatures can achieve the efficiency ranges from 35% to 38%.

- The efficiency of the super critical power plants operating at 220 bar steam pressure, 600/600 ℃ superheat/reheat temperatures can achieve is 42%.

- The efficiency in the range of 45 to 48% can be achieved by the ultra-super critical power plants operating at 300 bar pressure, 600/600 ℃ superheat/reheat steam temperatures.

### Advantages and Disadvantages

The advantages and disadvantages of steam power plant are as follows:

#### Advantages:

- As compared with power generating plant, it has low initial cost and hence economical.

- Less land area is required as compared with the hydro power plant.

- Coal is used as fuel and the cost of coal is cheaper than petrol and diesel fuel. So the power generation cost is economical.

- This power plant has easy maintenance cost.

- Steam power plant can be installed in any area where water sources and transportation facility are easily available.

#### Disadvantages:

- The running cost of steam power plant is comparatively high because of fuel, maintenance etc

- If we talk about the overall efficiency of steam power plant, than is about 35 % to 41% which is low.

- Due to the release of burnt gases of the coal or fuel, it contributes to the global warming to a larger extent.

- The heated water that is thrown in the rivers, ponds etc puts and adverse effect on the living organism of water and disturbs the ecology.

“Thank You Teacher for Helping Me Bloom”